Author: Eugenio M. Tait

Graduated in Philosophy

Engineer in Electricity and Electronics

E-mail: eugeniotait@hotmail.com

Web: http://www.geocities.com/tdcee

Web: http://www.geocities.com/eugeniomtait

Chap. 09	Amplification of Audiofrecuency in low level class A

Previous theory of the TBJ

Previous theory of the JFET

General characteristics of operation

Bipolar transistor of juncture TBJ

Common emitter

Base common

Common collector

Transistor of effect of juncture field JFET

Common source

Common gate

Common drain

Design common emitter

Design common base

Design common collector

Design common drain

Adds with AOV

Design

________________________________________________________________________________

Previous theory of the TBJ

Once polarized the transistor, we can understand their behavior with the hybrid parameters. In continuous and common emitter is (h21E = )

VBE  =  h11E IB + h12E VCE

IC     =  h21E IB + h22E VCE

and dynamically

VBE  =  h11e IB + h12e VCE

IC     =  h21e IB + h22e VCE

where

h11e   =  VBEIB

h12e   =  VBEVCE 	~  0

h21e   =  ICIB	~  

h22e   =  ICVCE

or with a simpler terminology

vbe  =  h11e ib + h12e vce	    ~  h11e ib

ic    =  h21e ib + h22e vce     ~  h21e ib

It will be useful also to keep in mind the transconductance of the dispositive

gm  =  ic / vbe  ~  h21e /  h11e

We know that these parameters vary with respect to the polarization point, temperature and frequency. Inside a certain area, like sample the figure, we will be able to consider them almost constant.

To measure the parameters of alternating of the simplified transistor, that is to say rejecting h12e and h22e, we can appeal to the following circuit, where will have a short-circuit in the collector if we design

h22e-1   >>   RC  = ...      100 []

and we measure with an oscilloscope for the polarization wanted without deformation (to remember that when exciting with voltage the sign it will be small, because the lineality is only with the current)

VCE  = ...

IC  =  ( VCC - VCE ) / RC  = ...

vR0p  = ...

vcep  = ...

vbep  = ...

being with it

h11e  =  vbep / ibp  =  R0  vbep / vR0p  = ...

h21e  =  icp / ibp  =  R0  vcep / RC  vR0p  = ...

gm  =  icp / vbep  = ...

Previous theory of the JFET

Once polarized the JFET one has that

ID  =  Gm VGS + Gds VDS

and taking increments

ID  =  gm VGS + gds VDS

gm  =  IDVGS

gds-1  =  rds  =  VDSID

or with another more comfortable nomenclature

id  =  gm vgs + gds vds

and if we maintain constant ID we find the amplification factor 

0  =  gm vgs + gds vds

  =  - vds / vgs = gm rds

As in general rds it is big and worthless in front of the resistances in the drain below the 10 [K], it is preferred to use the simplification

id  ~  gm vgs

A practical circuit for the mensuration of the transconductance gm is the following one, where it is considered to the connected drain to earth for alternating

rds   >>   RD  = ...      100 []

and we measure with an oscilloscope

VDS  = ...

ID  =  ( VCC - VDS ) / RD  = ...

vdsp  = ...

vgsp  = ...

being with it

gm  =  idp / vgsp  =  RD  vdsp / RC  vgsp  = ...

General characteristics of operation

The methodology that will use responds to the following circuit

Bipolar transistor of juncture TBJ

Common emitter

Zent  =  vent / ient  =  vbe / ib  =  h11e

Av  =  vsal / vent  =  - ic ZC / ib h11e =  - gm ZC

Ai  =  isal / ient  =  - ( vsal / ZC ) / ( vent / Zent )  =  - Av Zent / ZC  =  h21e

Zsal  =  vx / ix  =  ZC

Common base

Zent  =  vent / ient  =  veb / ie  =  ib h11e / ( ib + ib h21e )  =  h11e / ( 1 + h21e ) ~ gm-1

Av  =  vsal / vent  =  ic ZC / ib h11e =  gm ZC

Ai  =  isal / ient  =  - Av Zent / ZC  =  h21e / ( 1 + h21e )  ~  1

Zsal  =  vx / ix  =  ZC

Common collector

Zent  =  vent / ient  = [ibh11e + (ib + ib h21e) ZC] / ib = h11e + (1 + h21e) ZC ~ h11e+ h21e ZC

Av  =  vsal / vent  =  ie ZC / ib Zent  =  [1 + h11e / (1 + h21e) ZC ]-1     1

Ai  =  isal / ient  =  - Av Zent / ZC  =  1 + h21e  ~  h21e

Zsal  =  vx / ix  =  ZC //  [ ib (h11e + Zg ) / ie ]  ~  ZC //  [ ( h11e + Zg ) / h21e ]

Transistor of effect of juncture field JFET

The considerations are similar that for the TBJ but with rgs = h11e = .

Common source

Zent  =  vent / ient  =  

Av  =  vsal / vent  =  - gm ZC

Ai  =  isal / ient  =  (no entra corriente)

Zsal  =  vx / ix  =  ZC

Common gate

Zent  =  vent / ient  =  gm-1

Av  =  vsal / vent  =  gm ZC

Ai  =  isal / ient  =  1

Zsal  =  vx / ix  =  ZC

Common drain

Zent  =  vent / ient  =  

Av  =  vsal / vent    1

Ai  =  isal / ient  =  (no entra corriente)

Zsal  =  vx / ix  =  ZC //  gm-1

Design common emitter

Interested only in dynamic signs, the big capacitances of the circuit will maintain their voltages and they are equivalent to generators of ideal voltage with a value similar to the one that it have in their polarization.

The same as like it has been made previously we find

Av = vL/vg = - ic (RC//RL) / ig [Rg+(h11e//RB//RS)] = - gm (RC//RL) / [1+Rg/(h11e//RB//RS)]

Ai = iL/ig = - ic[(RC//RL)/RL]/ ib[h11e/(h11e//RB//RS)] = - h21e/ (1+RL/RC)[1+h11e/(RB//RS)]

Zent  =  vg/ig = ig [Rg+(h11e//RB//RS)]/ig = Rg+(h11e//RB//RS)

Zsal  =  vsal/isal = RC//(vsal/ic) = RC

If the following data are had

Avmin  = ...   Rg  = ...   RL = ...   fmin  = ...

we choose a TBJ and of the manual or their experimentation we find

VCE  = ...   IC  = ...     = ...   h21e  = ...   h11e  = ...   gm  =   h21e / h11e  = ...

Keeping in mind that seen in the polarization chapter adopts

VRE  = ...    1 [V]

1    SI  = ...    20

originating

RC  =  VCE  / IC  = ...

RE  ~  VRE  / IC  = ...

VCC  =  2 VCE  + VRE  = ...

RB  =  ( SI  - 1 ) RE VCC / [ 0,6 + VRE + ( SI - 1 ) RE IC -1 ]  = ...

RS  =  { [ ( SI - 1 ) RE ]-1 - RB-1 }-1  = ...

and we verify the gain

gm (RC//RL) / [1+Rg/(h11e//RB//RS)]  = ...      Avmin

So that the capacitances of it coupled they don't present comparable voltage in front of the resistance that you go in their terminals, it is

RL   >>   1 / min CC          CC  = ...   >>  1 / min RL

h11e//RB//RS   >>   1 / min CB          CB  = ...   >>  1 / min h11e//RB//RS

and that of disacoupled

h11e   >>   (1+h21e) . 1 / min CE

that it will usually give us a very big CE. To avoid it we analyze the transfer of the collector circuit better with the help of the power half to min, that is to say ~ 3 [dB]

vc /vb  =  ic(RC//RL) / [ibh11e+ (ic+ib)ZE] ~ 0,707 gm(RC//RL)

of where then

CE  = ...   >   ( h21e2 + 2h21eh11e/RE )1/2 / h11emin

Design common base

If we call using Thevenin

RT  =  Rg // RE

vT  =  vg RE / ( RE + Rg )

the same as like we have made previously we can find

Av = vL/vg = ic (RC//RL) / vT [1+(Rg/RE)] ~ gm (RC//RL) / [1+Rg/(RE//gm)]

Zent  =  vg/ig = Rg+[RE//(veb/ie)] ~ Rg+ (RE//gm)

Ai = iL/ig = Av Zent / RL = (1+RL/RC) (1+ 1/REgm )

Zsal  =  vsal/isal = RC//(vsal/ic) = RC

If the following data are had

Avmin  = ...   Rg  = ...   RL = ...   fmin  = ...

we choose a TBJ and of the manual or their experimentation we find

VCE  = ...   IC  = ...     = ...   h21e  = ...   h11e  = ...   gm  =   h21e / h11e  = ...

Keeping in mind that explained in the polarization chapter, we adopt

VRE  = ...    1 [V]

1    SI  = ...    20

originating

RC  =  VCE  / IC  = ...

RE  ~  VRE  / IC  = ...

VCC  =  2 VCE  + VRE  = ...

RB  =  ( SI  - 1 ) RE VCC / [ 0,6 + VRE + ( SI - 1 ) RE IC -1 ]  = ...

RS  =  { [ ( SI - 1 ) RE ]-1 - RB-1 }-1  = ...

and we verify the gain

gm (RC//RL) / [1+Rg/(RE//gm)]  = ...      Avmin

So that the capacitances of it coupled they don't present comparable voltage in front of the resistance among their terminals it is

RL   >>   1 / min CC          CC  = ...   >>  1 / min RL

and that of desacoupled

h11e   >>   1 / min CB         CB  = ...   >>  1 / min h11e

and in a similar way we reason with the condenser of the emitter

RE // gm   >>   1 / min CE

but that it will usually give us a very big CE. To avoid it we analyze the transfer better from the collector circuit to base with the power half to wmin, that is to say ~ 3 [dB]

vc /vent  =  ic(RC//RL) / [ibh11e+ (ic+ib)ZE] ~ 0,707 gm(RC//RL)

of where then

CE  = ...   >   1 / min gm

Design common collector

If we call using Thevenin

RT  =  Rg // RB  // RS

vT  =  vg ( RB  // RS ) / ( Rg + RB  // RS )

the same as like it has been made previously we find

Av = vL/vg = ie (RE//RL) / vT [ (Rg+RB //RS) / (RB //RS) ] ~

~ [h21e RE//RL/(h11e+h21eRE//RL)] . {(1+Rg/RB //RS) [1+RT/(h11e+h21eRE//RL)]}-1 ~ 1

Zent  =  vg/ig = Rg+RB//RS//[(ibh11e+ieRE//RL)/ib] ~ Rg+ RB//RS//(h11e+h21eRE//RL)

Ai = iL/ig = Av Zent / RL  h21e

Zsal  =  vsal/isal = RE//[ib(h11e+RT)/ie] ~ RE//[(h11e+Rg//RB//RS)/h21e]

If the following data are had

Rent  = ...   Rg  = ...   RL = ...   fmin  = ...

we choose a TBJ and of the manual or their experimentation we find

VCE  = ...   IC  = ...     = ...   h21e  = ...   h11e  = ...   gm  =   h21e / h11e  = ...

Keeping in mind that seen in the polarization chapter adopts

VRE  =   VCE  = ...    1 [V]

1    SI  = ...    20

originating

RE  ~  VRE  / IC  = ...

VCC  =  2 VCE  = ...

RB  =  ( SI  - 1 ) RE VCC / [ 0,6 + VRE + ( SI - 1 ) RE IC -1 ]  = ...

RS  =  { [ ( SI - 1 ) RE ]-1 - RB-1 }-1  = ...

and we verify the input resistance

Rg+ RB//RS//(h11e+h21eRE//RL)  = ...      Rent

So that the capacitances of it couples they don't present comparable voltage in front of the resistance in their terminals, it is

RL   >>   1 / min CE          CE  = ...   >>  1 / min RL

Rent  >>   1 / min CB          CB  = ...   >>  1 / min [Rg+ RB//RS//(h11e+h21eRE//RL)]

Design common drain

Generally used to adapt impedances, that is to say with the purpose of not loading to the excitatory generator, the following circuit will be the one proposed. Let us find their main ones then characteristic

Zent  =  vg / ig  =  Rg + RG

Zsal  =  vsal / isal  =  RE // ( vsg/id )  =  RE // gm

If then we have the following data

Rent  = ...   Rsal  = ... Rg  = ...   RL = ...   fmin  = ...

we choose a JFET and of the manual or their experimentation we find

VP  = ...   IDSS  = ...   IG0  = ...

If we keep in mind that Rsal = gm-1 in this configuration, and that

gm  =  IDVGS  ~  2 IDSS  ( 1 + VGS / VP ) / VP

we make

0  >  VGS  = ...  VP (VP 2 IDSSRsal ) ] - 1   = ...

ID  =  IDSS  ( 1 + VGS / VP )2  = ...

VDS  = ...     VP

VCC  =  VDS - VGS  = ...

RS  =  - VGS / ID  = ...

Rent     RG  = ...   <<   - VGS / IG0

We design the capacitors of it coupled so that to the minimum frequency they have worthless reactance

RL   >>   1 / min CS          CS  = ...   >>  1 / min RL

RG  >>   1 / min CG          CG  = ...   >>  1 / min RG

Adds with AOV

Commonly to this circuit it denominates it to him mixer. In the following figure we see a possible implementation, where it is observed that it is not more than an amplifier inverter of «n» entrances, and that it possesses a filter of high frequencies in their feedback circuit.

Their behavior equations are

R1  =  Rg1 + R1n

Rent n  =  R1n

Avn  =  Z2 / R1

vL  =  ( vg1 + vg2 + ... vgn ) Avn  =  - ( vg1 + vg2 + ... vgn )  ( R2 // sC-1 ) / R1

Design

Be the data

Rg1  = ...    Rg2  = ...    Rgn  = ...

fmax  = ...   Av0  = ... (minimum gain in the band pass)

We choose an AOV and of the manual or their experimentation we obtain

VCC  = ...   IB  = ...  (con JFET IB = 0)

therefore

R2  = ...   <<   VCC / 2 IB

R1n  =   ( R2 / Av0 ) - Rgn  = ...

R3  ~   R2 //  ( R1 / n )  =  R2 //  [ ( R2 / Av0 ) / n ]  = ...  (con JFET R3 = 0)

If we design the capacitor so that it produces the power half to the minimum specified frequency

Av (max)  =  0,707 Av0  =  R2 / R1 [ 1 + ( max C R2 )2 ]1/2   C  =  1 / maxR2   = ...

As for any AOV the maximum advisable power is of the order of 0,25 [W], we prevent

0,25  >  VCC2 / RL   RL   = ...     1 [K]

_________________________________________________________________________________