Author: Eugenio M. Tait

Graduated in Philosophy

Engineer in Electricity and Electronics

E-mail: eugeniotait@hotmail.com

Web: http://www.geocities.com/tdcee

Web: http://www.geocities.com/eugeniomtait

Chap. 41	Controllers of the State in a System

CONTROLLERS TYPE «P.I.D.»

INTRODUCTION

Optimization for Ziegler-Nichols

First form

Second form

COMPENSATORS TYPE «DEAD-BEAT» AND «DAHLIN»

Compensator «dead-beat»

Compensator «Dahlin»

CALCULATION OF A CONTROLLER COMPESATOR

DATA

PHYSICAL IMPLEMENTATION

GENERALITIES

CALCULATION

DIAGRAM OF FLOW

_________________________________________________________________________________

CONTROLLERS TYPE «P.I.D.»

INTRODUCTION

The transfer of the compensator Gc is given in the following way

Gc(s)	=  Kp [ 1 + Tds + 1/Tis ]	=   Kp + Kds + Ki/s

where

Kp	=   proportion gain

Kd	=   derivation gain

Ki	=   integration gain

Td	=   constant of derivative time

Ti	=   constant of integration time

Optimization for Ziegler-Nichols

It consists on two methods to calculate the Gc in such a way that the over-impulse in y(t) it doesn't overcome 25% for an input r(t) in pedestal.

First form

We apply a pedestal in u(t) and they are experimentally for the plant Gp

T0	=   ...	Delay time

	=   ...	Constant of time

and if approximately for the graph a plant transfer

Gp(s)	=  K . e-sTo / ( 1 + s  )

some values are suggested for the design

P		PI		PID

Kp		 / T0		0,9 .  / T0	1,2 .  / T0

Td		0		0		0,5 . T0

Ti				3,33 . T0	2 . T0

that is to say that finally is for our case of PID

Gc(s)	=  0,6 .  ( s + 1/ T0 )2 / s

Second form

We apply this method to those plants Gp that have harmonic oscillations in closed loop Glc when it experiences them to him with a proportional compensator Gc(s)= Kp.

We find the critical value that makes oscillate the plant and their period

Kpc	=   ...	Kp critical

T0	=   ...	period of critical oscillation

and it is suggested to use the values

P		PI		PID

Kp		0,5 . Kpc	0,45 . Kpc	0,6 . Kpc

Td		0		0		0,125 . T0

Ti				0,83 . T0	0,5 . T0

that is to say that finally is for our case of PID

Gc(s)	=  0,075 . Kpc . T0 ( s + 4/ T0 )2 / s

COMPENSATORS TYPE «DEAD-BEAT» AND «DAHLIN»

Compensator «dead-beat»

Given the figure of closed loop Glc, we look for to design the compensator D(z) such that the output follows the most possible to the input; that is to say, that diminishes the error «e»

y(k)	=   r(k-1)

With the purpose of simplifying the nomenclature we will use the following expressions:

Gp(z)	=   y(z) / u(z)	=   Z [Go(s).Gp(s)]   =   Z (1 - e-sT) . Z [Gp(s) / s]

D(z)	=   u(z) / e(z)	=   Z [D(s)]

Then, first we outline the transfer of closed loop

Glc(z)	=   y(z) / r(z)	=   D(z) . Gp(z)  /  [ 1 + D(z) . Gp(z) ]

and we clear

D(z)	=   [ 1 / Gp(z) ]  . {  Glc(z)  /  [ 1 - Glc(z) ]  }

If now we keep in mind the transformation of the impulse of Kronecker in the sample moment «k»

Z [(k)]	   =   z-k

and that for the equation y(k) = r(k-1) it is in the practice that for a delay (retard of Glc) of «n» pulses (the sample «k» a sample will be for above for the maxim possible response of Glc)

k	=   n + 1

y(k)	=   r((n+1)-1)	=   r(n)

of where we obtain

y(z)	=   r(z) . z-1		para n = 0

y(z)	=   r(z) . z-(n+1)		para n  0

Glc(z) 	=   y(z) / r(z)	=   z-(n+1)	=   z-n-1

what determines finally

D(z)	=   [ 1 / Gp(z) ] . { z-n-1/ [1-z-n-1] }

Compensator «Dahlin»

Here they will diminish the undesirable over-impulses in exchange for allowing a worsening in the error «e».

For it Dahlin proposes the following algorithm

y(k)	=   q . y(k-1) + ( 1 - q ) . r(k -(n+1))

where «q» it is defined as a «syntony factor»

0    q  =  e -T    1

and what allows to determine (to observe that the expression coincides with that found in «dead-beat» for )

Glc(z) 	=   y(z) / r(z)	=   ( 1 - q ) . z-n-1 / ( 1 - q.z-1)

D(z)	=   [ 1 / Gp(z) ] . { ( 1 - q ) . z-n-1 / [ 1 - q.z-1- ( 1 - q ) . z-n-1 ] }

CALCULATION OF A CONTROLLER COMPESATOR

DATA

We have the following servomechanism position controller, and it is wanted him not to possess oscillations to their output. It is asked to use a digital processor of control Gc to their input that avoids the effect.

B	=	0,15 [Nms/r]

J	=	0,15 [Nms2/r]

R	=	1 []

L	=	0,03 [H]

Nnom	=	1500 [RPM]

Pnom	=	3/4 [Hp]

Unom	=	200 [V]

Inom	=	3 [A]

A	=	10

K	=	1 [V/º]

n	=	1/10

t		0,1 [s]

Oscillations died in  (t)

PHYSICAL IMPLEMENTATION

GENERALITIES

We determine the characteristics of the motor

nom	=	2 . N / 60   =   157 [r/s]

Pnom	=	3/4 [Hp] / 740   =   555 [W]

kg	~	Unom / nom   =   1,27 [Vs/r]

and finding the poles of the plant

elec	=	L / R   =  0,03 [s]

mec	=	J / B   =  1 [s]			dominant

we make their transfer

Gp(s)	=	y(s) / u(s)   ~   A . ( / U) / s ( 1 + s mec )   =

=	A.kg-1 / s ( 1 + s mec )  ~   15,7 [r/V] / s ( 1 + s )

The sampling frequency obtains it of the Theorem of the Sampling

T	»	t

for what is adopted for example

T	=	1 [s]

CALCULATION

Given the gain of the R.O.C.

Go(s)	=   1 - e-sT / s

it is the plant Gp

Gp(z)   	=   Z [ Go(s)Gp(s) ]  =  Z ( 1 - e-sT ) . Z [ 15,7 / s2 ( 1 + s ) ]  =

=   5,65 (1+0,71z-1) z-1 / (1-z-1) (1-0,36z-1)

If we outline an input generic type pedestal

r(t)	=   U

r(z)	=   1 / ( 1 - z-1 )

the signal of control u(z) it is

u(z)	=   Glc . r / Gp   =   Glc .  (1-0,36z-1) / 5,65z-1(1+0,71z-1)

and like we know that the order of Glc will be smaller than 3 to avoid oscillations, and that in turn it will be same or bigger that that of Gp, we can conclude here that it is correct that it has two poles

Glc(z)	=   a z-1 + b z-2

and in turn if to simplify calculations we also make

Glc(z)	=   K z-1(1+0,71z-1)

they are

K	=   (a z-1 + b z-2) / z-1(1+0,71z-1)   =   a + RK [ (b-0,71a)z-1 ]

u(z)	=   0,18 K (1-0,36z-1)

If now we outline the closed loop Glc again

Glc(z)	=   y / r   =   ( r - e ) / r   =   GcGp / ( 1 + GcGp )

and we clear the error e(z)

e(z)	=   ( 1 - Glc ) r   =   ( 1 - Glc ) / ( 1 - z-1 )

that we know it will be a polynomial N(z)

e(z)	=   N(z)

we can deduce

N(z)	=   ( 1 - Glc ) / ( 1 - z-1 )   =  [ 1 - ( a z-1 + b z-2 ) ] / ( 1 - z-1 )   =

=   1 + (1-a)z-1 + RN [ (1-a-b)z-2 ]

If now of the two equations of K and N we adopt null the remains

RK	=   RN  =   0

we can calculate

a	=   0,58

b	=   0,41

K	=   0,58

Glc(z)	=   0,58 z-1 + 0,41 z-2

N(z)	=   1 + 0,41 z-1

as well as if we outline again

Glc(z)	=   GcGp / ( 1 + GcGp )

e(z)	=   N(z)   =   ( 1 - Glc ) r   =   ( 1 - Glc ) / ( 1 - z-1 )

we clear the filter Gc controller finally

Gc(z)	=   Glc / Gp ( 1 - Glc )   =   Glc / Gp N(z) ( 1 - z-1 )   ~

~   0,11 (z-0,36) / (z+0,41)

what will give us an output and an error

y(z)	=   Glc. r   =   (0,58z-1 + 0,41z-1) / (1-z-1)   =   0,58 z-1 + z-2 + z-3 + ...

u(z)	=   0,18.0,58 (1-0,36z-1)   =   0,1 - 0,037z-1

and in the time

y(k=0) = 0   	y(k=1) = 0,58   	y(k=2) = 1   	y(k=3) = 1   	y(k=4) = 1

u(k=0) = 0,1   	u(k=1) = - 0,037   u(k=2) = 0   	u(k=3) = 0   	u(k=4) = 0

To implement the filter digital controller we outline their transfer first

Gc(z)	=   u / e   =   0,11 (z-0,36) / (z+0,41

and we proceed

z u + 0,41 u   =   0,11 z e - 0,04 e

that is to say that is for a k-generic instant

u(k+1) + 0,41 u(k)   =   0,11 e(k+1) - 0,04 e(k)

or

u(k) + 0,41 u(k-1)   =   0,11 e(k) - 0,04 e(k-1)

what will determine us a control u(k) to implement in the following way

u(k)	=   0,11 e(k) - 0,04 e(k-1) - 0,41 u(k-1)

DIAGRAM OF FLOW

_________________________________________________________________________________